Faculty of Engineering Ain Shams University

ECE413s - ASIC Design and Automation

Fall 2024

Traffic Light Controller

| Name | ID | Task |
| --- | --- | --- |
| Mustafa Ahmed Hashem | 2100467 | Controller |
| Kareem Ehab Ibrahim | 2100913 | Test bench |
| Michael George Naem | 2100709 | FSM |
| Aya Faried Hussain | 2001967 | Light algorithm |

# Traffic Light Controller System Report

## System Design

This Traffic Light Controller system is designed to manage traffic flow at a four-way intersection. The design focuses on efficient traffic regulation by dynamically adjusting light durations based on real-time sensor inputs. The system ensures fairness and reduces congestion by prioritizing active lanes while maintaining safe transitions between signals. The key design considerations include:
- Scalability: The system can adapt to different configurations and input parameters.
- Responsiveness: Real-time sensor inputs enable dynamic adjustments to traffic light durations.
- Safety: A robust state machine ensures safe transitions and prevents conflicts.

## Light Algorithm

The traffic light algorithm determines the behavior of each light at the intersection. The system operates in four main phases corresponding to the four directions (North, South, East, West). Each phase includes green, yellow, and red states, with durations influenced by sensor inputs. The algorithm prioritizes lanes with active traffic (indicated by sensor_1th signals) and extends green durations if extended traffic is detected (sensor_5th signals). The sequence ensures all directions are served equitably.

## Controller Specifications

The controller operates on a finite state machine (FSM) with the following parameters:
- DEFAULT_GREEN_TIME: The base green light duration is set to 9 time units.
- EXTENDED_GREEN_TIME: If extended traffic is detected, the green light duration is increased to 18 time units.
- YELLOW_TIME: A fixed duration of 2 time units for the yellow light ensures safe transitions.
- RED_TIME: A fixed duration of 2 time units ensures all lights are red before transitioning to the next green phase.
Inputs:
- clk: Clock signal to synchronize operations. (recommend clock 5 second)
- rst: Reset signal to initialize the system to a safe state.
- sensor_1th: 4-bit input indicating the presence of traffic in each direction.
- sensor_5th: 4-bit input indicating extended traffic in each direction.
Outputs:
- Light_north, Light_south, Light_east, Light_west: 2-bit signals representing the state (red, yellow, green) of lights in each direction.

## FSM Design (Hybrid state machine -> Moore for Light & Mealy for timer)

The Finite State Machine (FSM) for the traffic light controller has the following states:
- ALL_RED: All lights are red; this is the default and safe state during reset or transitions.
- TLNG, TLEG, TLSG, TLWG: Green light states for North, South, East, and West directions.
- TLNY, TLEY, TLSY, TLWY: Yellow light states for respective directions.
State transitions are determined by:
- Sensor inputs indicating active traffic.
- Timer values to ensure adequate duration for each phase.
The FSM ensures that no two conflicting directions are green simultaneously and maintains fair rotation among directions.

They both describe the system accurately but the handwritten is more detailed

Notes:

-Green time depends on the Sensor 1st & 5th

-Any outputs aren’t mentioned are by default 0

Red = 0, Green = 2, Yellow =1

## Testbench

The testbench simulates the Traffic Light Controller to validate its functionality under different scenarios. The testbench includes:
- Clock Generation: A periodic clock signal drives the FSM.
- Reset Conditions: The system is initialized to the ALL_RED state for safety.
- Sensor Inputs: Stimuli simulate the presence and intensity of traffic in each direction.
Test cases include:
1. No sensors active: Verifies the default behavior.
2. Single direction active: Validates green light activation for active lanes.
3. Extended green duration: Confirms longer green light periods when extended traffic is detected.
4. Multiple directions active: Ensures proper prioritization and rotation among active lanes.
5. Reset: Tests the FSM's ability to return to a safe state after reset.

## Simulation Results

Simulation results demonstrate the correct operation of the Traffic Light Controller. Key observations include:
- Proper transitions between green, yellow, and red phases.
- Dynamic adjustment of green light durations based on sensor inputs.
- Safe operation under reset and conflict scenarios.
The system successfully meets the design objectives, ensuring efficient and safe traffic management.

## Verilog Code

The following is the Verilog code for the Traffic Light Controller and its testbench:

module trafficLightController(

input clk,                  // Clock signal

input rst,                  // Reset signal

input [3:0] sensor_1th,     // Sensors to select active traffic light

input [3:0] sensor_5th,     // Sensors to extend green duration

output reg [1:0] Light_north ,Light_south,Light_east,Light_west

);

// Timing Parameters

parameter DEFAULT_GREEN_TIME = 4'd9;

parameter EXTENDED_GREEN_TIME = 5'd18;

parameter YELLOW_TIME = 4'd2;

parameter RED_TIME = 4'd2;

// State Parameters

parameter ALL_RED = 4'b0000,

TLNG = 4'b0001, TLEG = 4'b0010, TLSG = 4'b0100, TLWG = 4'b1000,

TLNY = 4'b1001, TLEY = 4'b1010, TLSY = 4'b1011, TLWY = 4'b1100;

reg [3:0] current_state, next_state; // Current and next state

reg [4:0] timer; // Timer for light durations

// State transition logic

always @(posedge clk or posedge rst) begin

if (rst) begin

current_state <= ALL_RED;

timer <= 4'd0;

//{Light_north,Light_east,Light_south,Light_west} = 8'b00000000; // All lights off

end else if (timer == 0) begin

current_state <= next_state;

case (next_state)

TLNG: timer <= (sensor_1th[0] & sensor_5th[0]) ? EXTENDED_GREEN_TIME : DEFAULT_GREEN_TIME;

TLEG: timer <= (sensor_1th[1] & sensor_5th[1]) ? EXTENDED_GREEN_TIME : DEFAULT_GREEN_TIME;

TLSG: timer <= (sensor_1th[2] & sensor_5th[2]) ? EXTENDED_GREEN_TIME : DEFAULT_GREEN_TIME;

TLWG: timer <= (sensor_1th[3] & sensor_5th[3]) ? EXTENDED_GREEN_TIME : DEFAULT_GREEN_TIME;

TLNY, TLEY, TLSY, TLWY: timer <= YELLOW_TIME;

ALL_RED: timer <= RED_TIME;

endcase

end else begin

timer <= timer - 1; // Decrement timer

end

end

// Next state logic for fair rotation

always @(*) begin

case (current_state)

ALL_RED: begin

if (sensor_1th[0]) next_state = TLNG;

else if (sensor_1th[1]) next_state = TLEG;

else if (sensor_1th[2]) next_state = TLSG;

else if (sensor_1th[3]) next_state = TLWG;

else next_state = TLNG; // Default start

end

// Green to Yellow transitions

TLNG: next_state = TLNY;

TLEG: next_state = TLEY;

TLSG: next_state = TLSY;

TLWG: next_state = TLWY;

// Yellow to next Green transitions

TLNY: next_state = TLEG;

TLEY: next_state = TLSG;

TLSY: next_state = TLWG;

TLWY: next_state = TLNG;

default: next_state = ALL_RED; // Safety fallback

endcase

end

// Traffic light outputs

always @(*) begin

case (current_state)

TLNG: {Light_north,Light_east,Light_south,Light_west} = 8'b10000000; // North green

TLEG: {Light_north,Light_east,Light_south,Light_west} = 8'b00100000; // South green

TLSG: {Light_north,Light_east,Light_south,Light_west} = 8'b00001000; // East green

TLWG: {Light_north,Light_east,Light_south,Light_west} = 8'b00000010; // West green

TLNY: {Light_north,Light_east,Light_south,Light_west} = 8'b01000000; // North yellow

TLEY: {Light_north,Light_east,Light_south,Light_west} = 8'b00010000; // South yellow

TLSY: {Light_north,Light_east,Light_south,Light_west} = 8'b00000100; // East yellow

TLWY: {Light_north,Light_east,Light_south,Light_west} = 8'b00000001; // West yellow

default: {Light_north,Light_east,Light_south,Light_west} = 8'b00000000; // All red

endcase

end

endmodule

// Test Bench

`timescale 1s/1ms

module trafficLightController_tb;

// Inputs

reg clk;

reg rst;

reg [3:0] sensor_1th;

reg [3:0] sensor_5th;

// Outputs

wire [1:0] Light_north ,Light_south,Light_east,Light_west;

// Instantiate the trafficLightController module

trafficLightController uut (

.clk(clk),

.rst(rst),

.sensor_1th(sensor_1th),

.sensor_5th(sensor_5th),

.Light_north(Light_north),

.Light_east(Light_east),

.Light_west(Light_west),

.Light_south(Light_south)

);

// Clock generation

initial begin

clk = 0;

forever #2.5 clk = ~clk; // Clock with 10-time unit period

end

// Test sequence

initial begin

// Monitor changes

$monitor("Time=%0d | rst=%b | sensor_1th=%b | sensor_5th=%b | Light_north=%b | Light_south=%b | Light_east=%b | Light_west=%b |", $time, rst, sensor_1th, sensor_5th, Light_north ,Light_south,Light_east,Light_west);

// Test case 1: No sensors active

rst = 1; sensor_1th = 4'b0000; sensor_5th = 4'b0000;

#100 rst = 0;

// Test case 2: North light active

#200 sensor_1th = 4'b0001; sensor_5th = 4'b0000;

// Test case 3: North light extended green

#200 sensor_1th = 4'b0001; sensor_5th = 4'b0001;

// Test case 4: South light active

#400 sensor_1th = 4'b0011; sensor_5th = 4'b0000;

// Test case 5: South light extended green

#400 sensor_1th = 4'b1100; sensor_5th = 4'b1010;

// Test case 6: Multiple sensors active (priority North)

#400 sensor_1th = 4'b1011; sensor_5th = 4'b1011;

// Test case 7: Reset the module

#700 rst = 1; #10 rst = 0;

#100 $finish; // End simulation

end

endmodule

## Wave form

The following is the waveform snapshots for the Traffic Light Controller:

Testcase 1:

Testcase 2:

Testcase 3:

Testcase 4:

Testcase 5:

Testcase 6:

Testcase 7: